HPLC analysis of the two compounds

The mobile phase was acetonitrile (A) : water (B), with gradient elution: 0 - 2 min, 30% A; 2 - 8 min, 30 - 60% A; 8 - 16 min, 60 - 90% A; 16 - 26 min, 90 - 100% A; 26 - 45 min, 100% A; 45 - 47 min, 100 - 30% A; 47 - 54 min, 30% A. The detection wavelength was 208 nm and 254 nm; the flow rate was 1 mL/min; the column temperature was 30℃; the injection volume was 10 μL. HPLC chromatogram are shown in Figures 1 and 2.

Fig.1 The HPLC chromatogram of compound A (Retention time: 14.8 min, Purity 98%)

Fig.2 The HPLC chromatogram of compound B (Retention time: 20.0 min, Purity 97%)

The black line represents the detection wavelength of 208nm, and the red line represents 254nm.

Main steps and parameters of spectral testing

13 CNMR:

Model：Bruker 400MHz, Germany

Probe model and pulse：PROBHD: 5mm PABBO BB; PULPROG: zg30.

Test temperature:297.2K.

Steps: Generally, the sample is dissolved by ultrasonic treatment with deuterated chloroform or by heating, then centrifuged to obtain the supernatant for testing, lock the field, set the parameters, collect the information, and organize the results.

ESI:

Model：Thermo Scientific QE plus, The United States.

Instrument parameters：

Ion source                   ESI           APCI

Steps：

1. Take an appropriate amount of solution or powder, and dilute and dissolve it with organic solvents such as dichloromethane to the desired concentration.

2. Filter through a 0.22 μm microporous membrane (usually made of hydrophilic PTFE or nylon material).

3. Transfer the finally processed clear and transparent liquid to the dedicated injection bottle, cover with a pad cover, and place it in the automatic injector for analysis.