2.12 RT-qPCR for Detection of SLC1A5 and c-Myc Expression in rats

To detect the expression of SLC1A5 and c-Myc in rat EMs lesions. RT-qPCR were performed under the kit instruction. Firstly, extraction of Total RNA from Tissue: Take 10 mg of fresh tissue and add 500 μL of Buffer RL from the FastPure Tissue Total RNA Isolation Kit (Beyotime, China, Cat#: P1101).  Homogenize the sample using a glass homogenizer until no obvious tissue chunks remain. Transfer the lysed sample to the FastPure gDNA-Filer Columns III and centrifuge at 12,000 rpm for 30 seconds. Discard the Columns III, collect the filtrate in a collection tube, add 0.5 times the volume of anhydrous ethanol (Sigma-Aldrich, USA, Cat#: 100983), and mix thoroughly. Transfer the entire mixture to the FastPure RNA Columns III and centrifuge at 12,000 rpm for 30 seconds. Discard the filtrate.Add 700 μL of Buffer RW1 to the FastPure RNA Columns III, centrifuge at 12,000 rpm for 30 seconds, and discard the filtrate.Add 700 μL of Buffer RW2 to the FastPure RNA Columns III, centrifuge at 12,000 rpm for 30 seconds, and discard the filtrate.Add 500 μL of Buffer RW2 (pre-mixed with anhydrous ethanol) to the FastPure RNA Columns III, centrifuge at 12,000 rpm for 2 minutes. Carefully remove the column from the collection tube to avoid contact with the filtrate, which could cause contamination.Centrifuge the column at 12,000 rpm for 1 minute to prevent ethanol contamination. Carefully transfer the column to a new RNase-free collection tube. Add 30 μL of RNase-free ddH2O (Thermo Fisher Scientific, USA, Cat#: 10977-035) to the center of the column, incubate at room temperature for 3 minutes, and then centrifuge at 12,000 rpm for 1 minute to elute the RNA. An additional washing step can be performed by centrifuging a second time to elute further. Measure the RNA concentration and purity using a NanoDrop 2000 (Thermo Fisher Scientific, USA) [1].

Secondly, reverse Transcription of Total RNA to cDNA: cDNA synthesis was performed in a 20 μL reaction system. In a 200 μL Eppendorf tube (RNase-free), 500 ng of RNA, 2 μL of 5× gDNA wiper mix, and RNase-free water to a total volume of 10 μL were added. After reacting at 42°C for 2 minutes in a regular PCR machine, the following reagents were added to the mixture: 2 μL of 10× RT Mix, 2 μL of Hiscript III Enzyme Mix, 1 μL of Oligo(dT)20VN, 1 μL of Random hexamers, and 4 μL of RNase-free water. The mixture was mixed well and incubated at 37°C for 15 minutes, followed by 5 seconds at 85°C. The resulting cDNA was stored at -20°C [1].

The last step for RT-qPCR: A 20 μL reaction system was prepared. In a 100 μL Eppendorf tube (RNase-free), 5 μL of 5-fold diluted cDNA, 4.2 μL of nuclease-free water, 10 μL of 2× Taq Pro Universal SYBR qPCR Master Mix, 0.4 μL of forward primer (10 μM), and 0.4 μL of reverse primer (10 μM) were added. The prepared reagents were then transferred into an 8-well PCR strip, sealed, and placed into the CFX96 Touch 1855195 Real-Time Fluorescent Quantitative PCR System. The reaction parameters were set, and amplification was performed. The results were calculated using the 2-ΔΔCt method. The qPCR process includes the following steps: pre-denaturation at 95°C for 30 seconds, denaturation at 95°C for 10 seconds, annealing at 60°C for 30 seconds, extension at 95°C for 15 seconds, and 40 cycles of repetition. Melt curve analysis was performed by holding at 65°C for 60 seconds, then heating to 95°C and holding for 15 seconds. Specific primers used for PCR amplification were synthesized based on the sequences listed in Table 1 [1].

[1] Yao L, Wu J, Wang X, Wang N. LINC01134 Directly Binds and Regulates SLC1A5 Stability to Promotes Colorectal Cancer Progression. J Cancer. 2024; 15: 6135-6147. https://doi.org/10.7150/jca.100147