Supplementary 1.

Method of the analysis:

The samples were injected into a gas chromatograph (Agilent 7890A) equipped with a mass spectrometer (5975C), and the device was set to Electron Impact mode. The electron energy was set to 70 eV and the mass range of 50 - 500 m/z. A capillary column (HP5-MS 30 cm by 0.25 millimeter) was coated with a 0.25 µm film and used in the process. The injection chamber and the detector were set to 270 and 290 degrees Celsius, respectively. The operating temperatures were as follows:

The initial temperature of the column was set to 60 degrees Celsius and was risen by 5 degrees per minute to 280 degrees and was maintained at that temperature for 3 minutes.

The gas flow rate (Helium) was maintained at 1.2 milliliter/min.

The compound analysis was performed according to the operating duration and the spectral data obtained from the Wiley libraries.

The chemical composition of Rosa Foetida extract was assessed using a gas chromatography– mass spectrometry device (5977B GC/MSD, Agilent, Santa Clara, CA, USA). The DB-5 ms capillary column experimental conditions were as follows: length = 30 m, inner diameter = 0.25 mm, film thickness = 0.25 µm, gas carrier: Helium with purity ≥ 99.9995% (Sigma-Aldrich Co., St. Louis, MO, USA), gas flow rate: 1 mL/min. Briefly, 1 µL of the final solution prepared was injected into the device with an autosampler in a split 10:1 ratio. The temperature of the injector was set at 250 °C. The oven temperature was programmed to start from 50 °C (storage time of 1 min), then increase 8 °C/ min rate up to 120 °C (storage time of 1 min). Finally, the temperature was to increment at a 6 °C/min rate up to 250 °C in about 15 min. The total device running time was 47 min, with a solvent delay of 0–3 min. The sample components’ names, and structure were identified using the National Institute of Standards and Technology (NIST 11 Variant) database.

Chemical analysis

| No. | CAS registration number | English name | Area % | Molecular formula | Molecular structural formula |
| --- | --- | --- | --- | --- | --- |
| 1 | 000083-46-5 | beta-Sitosterol; 24-Ethylcholest-5-en-3beta-ol; | 6.80 | C 29 H 50 O |  |
| 2 | 000463-40-1 | Linolenic acid; 9,12,15-all-cis-Octadecatrieno | 6.38 | C 18 H 30 O 2 |  |
| 3 | 000057-10-3 | Palmitic acid; Hexadecanoic acid | 5.97 | C 16 H 32 O 2 |  |
| 4 | 000060-12-8 | Phenethyl alcohol; 2-Phenylethanol | 2.83 | C 8 H 10 O |  |
| 5 | 000060-33-3 | Linoleic acid; (Z,Z)-9,12-Octadecadienoic aci | 2.71 | C 18 H 32 O 2 |  |
| 6 | 000057-11-4 | Stearic acid; Octadecanoic acid | 2.46 | C 18 H 36 O 2 |  |
| 7 | 005989-02-6 | 2(4H)-Benzofuranone,5,6,7,7a-tetrahydro-6-hydroxy-4,4,7a-trimethyl-, (6S,7aR)-; 2(4H)-Benzofuranone,5,6,7, 7a-t | 1.19 | C 11 H 16 O 3 |  |
| 8 | 000059-02-9 | Vitamin E; D-alpha-Tocopherol; (2R)-3,4-Di | 1.12 | C 29 H 50 O 2 |  |
| 9 | 000620-02-0 | 5-Methyl furfural; 5-Methyl-2-furaldehyde | 1.05 | C 6 H 6 O 2 |  |
| 10 | 000544-63-8 | Myristic acid; Tetradecanoic acid; 1-Tridecane | 0.76 | C 14 H 28 O 2 |  |