SUPPORTING INFORMATION

A Combination of Native LC-MS Approaches for the Comprehensive Characterization of the Antibody-Drug Conjugate Trastuzumab Deruxtecan

Evolène Deslignière1,2,3, Hélène Diemer1,2,3, Stéphane Erb1,2,3, Pierre Coliat3,4, Xavier Pivot3,4, Alexandre Detappe3,4, Oscar Hernandez-Alba1,2,3, Sarah Cianférani1,2,3,*

1Laboratoire de Spectrométrie de Masse BioOrganique, Université de Strasbourg, IPHC, CNRS UMR 7178, 67087 Strasbourg, France

2Infrastructure Nationale de Protéomique ProFI – FR2048, 67087 Strasbourg, France

3Strasbourg Drug Discovery and Development Institute (IMS), University of Strasbourg, 67000 Strasbourg, France

4Institut de Cancérologie Strasbourg Europe, 67000 Strasbourg, France

*Correspondence: sarah.cianferani@unistra.fr (Sarah Cianférani)

TABLE OF CONTENTS:

EXPERIMENTAL

- Text S1: Detailed protocol for peptide mapping experiments.

- Text S2: Detailed protocol for rpLC-MS experiments.

RESULTS

- Figure S1: Mass deconvolutions of the reference mAb trastuzumab and ADCs (T-DXd, BV, T-DM1) obtained from SEC-nMS experiments.

- Figure S2: HICxSEC-nMS analysis of trastuzumab.

- Figure S3: CEX-nMS analysis of trastuzumab.

EXPERIMENTAL

Supplementary text 1. Detailed protocol for peptide mapping experiments.

Acid trypsin digestion – 20 µg of ADC were solubilized in 174 μL of BisTris 50 mM, 0.1% RapiGest (Waters) at pH 6.5. Disulfide reduction was performed with 5 mM TCEP for 30 min at 57°C. The enzyme was prepared by suspending 20 µg of trypsin (Promega) in 100 µL of H2O. Digestion was performed by adding 2 μL of trypsin, which corresponds to a 1:50 enzyme:substrate ratio, during 5 hours at 37°C. The reaction was stopped by adding 1 μL of TFA. RapiGest was eliminated by incubation at 37°C for 30 min and centrifugation at 10,000 g for 5 min.

Pepsin digestion – 20 µg of ADC were solubilized in 44 μL of H2O. Disulfide reduction was performed with 5 mM TCEP for 45 min at 57°C. 1M HCl was added to the ADC solution to a final pH of 1-2. Pepsin (Promega) was prepared in H2O to obtain a concentration of 0.2 µg/µL. Digestion was performed by adding 1 µL of pepsin solution, which corresponds to a 1:100 enzyme:substrate ratio. Samples were incubated at 37°C for 45 min. The reaction was stopped by heating at 95°C for 10 min.

LC-MS/MS experiments – Liquid chromatography conditions on nanoAcquity chromatography system. The analysis was performed using a nanoACQUITY Ultra-Performance-LC (UPLC, Waters). 150 ng of ADC digest were trapped on a Symmetry C18 pre-column (C18, 180 µm x 20 mm, 5 µm particle size, Waters) and the peptides were separated on an ACQUITY UPLC® BEH130 C18 separation column (C18, 75 µm × 250 mm, 1.7 µm particle size, Waters). The solvent system consisted of 0.1% formic acid in water (solvent A) and 0.1% formic acid in acetonitrile (solvent B). Peptide trapping was performed during 3 min at a flow rate of 5 µL/min with 99% A and 1% B and elution was performed at 60 °C at a flow rate of 400 nL/min from 8% to 35% of B in 38 minutes.

MS/MS conditions on Q Exactive plus mass spectrometer. For optimal nanoLC-MS/MS, the mass spectrometer was operated in positive mode, with the following settings: spray voltage 1800 V and capillary temperature 250°C. The MS scan had a resolution of 70000, the AGC target was 3x106 and the maximum IT was 50 ms on m/z [300-1800] range. The MS/MS scans had a resolution of 17500, the AGC target was 1x105 and the maximum IT was 100 ms with fixed first mass of 100 m/z and Isolation window of 2 m/z. Top 10 HCD was selected with intensity threshold of 5x104 and dynamic exclusion of 3 s. The normalized collision energy (NCE) was fixed at 27 V. The complete system was fully controlled by Thermo Scientific™ Xcalibur™ software. Raw data collected were processed and converted with MSConvert into .mgf peak list format.

Data interpretation – Peptide identification. Search engines MASCOT 2.6.2 algorithm (Matrix Science) was used. The search was performed against the sequence of the light and heavy chains of the ADC. Spectra were searched with a mass tolerance of 10 ppm for MS and 0.05 Da for MS/MS data. The search was made without enzyme specified, in order to allow the identification of any non-specific cleavage peptide. Variable modifications were specified: oxidation of methionine residues, pyro-glutamylation of the N-termini, deamidation of asparagine, isomerization of aspartic acid residues and drug-linker conjugation (C52 F H56 N9 O13 and C26 O8 N6 H34) on cysteine residues. Peptide identifications were validated with a minimal Mascot ion score of 30.

Quantification of amino acid modifications. Quantification was done by extraction of ion current, using BioPharma Finder v3.2 (Thermo Fisher Scientific) or Skyline (www.skyline.ms).

Supplementary text 2. Detailed protocol for rpLC-MS experiments.

## Sample preparation – For middle-up level experiments, fifty micrograms of mAbs were solubilized in 41 μL of 50 mM Na2HPO4 and 150 mM NaCl at pH 6.7. For middle-up experiments, 13 μL of IdeS protease enzyme at 4 units/μL (FabRICATOR® from Genovis, Sweden) were added and the mixture was incubated at 37°C for 30 min.

## LC-MS conditions – LC-MS analyses was performed using an Agilent 1200 series coupled to a maXis II (Bruker) Q-ToF based mass spectrometer. A volume equivalent to 5 μg of sample preparation was injected on a BioResolve™ RP mAb 2.1 mm ID and 150 mm length, pore size 450 Å and particule size 2.7 μm, polyphenyl column (Waters) set at 80 °C. The UV chromatograms were acquired with a DAD detector operating from 200 to 400 nm. Ultrapure water for mobile phase A (0.10% TFA) and acetonitrile for mobile phase B (0.08% TFA) were used to perform the chromatographic separation. The gradient was generated at a flow rate of 300 μL/min and the elution program started at 25% mobile phase B during 2 minutes to reach 44% in 38 minutes. Then, 5 minutes of washing step at 75% and finally 14 minutes of equilibration at 25% mobile phase B. A blank was injected between each sample in the same conditions.

## The mass spectrometer was operated in positive mode with a capillary voltage of 4.5 kV and CID value was set at 70 eV. Acquisitions were performed on the mass range 500 – 5 000 m/z. Calibration was performed using the singly charged ions produced by a solution of 2 g/L caesium iodide in 2-propanol/water (50/50 v/v). Data interpretation was performed by using Compass DataAnalysis v4.3 software (Bruker Daltonics). Since the molecular isotopic cluster was not fully resolved, average molecular masses were measured.

RESULTS

Figure S1. Mass deconvolutions of the reference mAb trastuzumab and ADCs (T-DXd, BV, T-DM1) obtained from SEC-nMS experiments.

Figure S2. HICxSEC-nMS analysis of trastuzumab. HIC-UV chromatograms obtained (A) at the intact level, or (B) at the middle-up level. mAb cartoons with colored dots were used to represent the modifications corresponding to each charge variant: isomerization of aspartate (isoAsp, iD, brown) and deamidation (+1, green).

Figure S3. CEX-nMS analysis of trastuzumab. CEX-UV chromatograms obtained (A) at the intact level, or (B) at the middle-up level. mAb cartoons with colored dots were used to represent the modifications corresponding to each charge variant: isomerization of aspartate (isoAsp, iD, brown) and deamidation (+1, green).